| ID | UDS diagnostic specifications. | MQ_state | MQ_type | MQ_Release | MQ_testability | MQ_safetyrelevant |
| --- | --- | --- | --- | --- | --- | --- |
| SW10-1 | # 1 Diagnostic | agreed | headline | n/a | n/a | false |
| SW10-3 | ## 1.1 Diagnostic protocol services | agreed | headline | n/a | n/a | false |
| SW10-191 | ### 1.1.1 Supported Interfaces | agreed | headline | n/a | n/a | false |
| SW10-192 | #### 1.1.1.1 CAN | agreed | headline | n/a | n/a | false |
| SW10-193 | The SCM shall use the NORMAL addressing scheme | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-194 | The SCM shall require the use of 29-bit extended CAN ID's | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-201 | The SCM shall have a maximum transport protocol message length of 4095 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-202 | The SCM shall have a tester present ID of 0x3E00 for physical connections | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-203 | The SCM shall have a tester present ID of 0x3E80 for functional connections | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1626 | ##### 1.1.1.1.1 JDM UDS Message Configuration | agreed | headline | n/a | n/a | false |
| SW10-195 | The SCM shall use the address 0x18DAF5FE for all JDM CAN-ID requests | agreed | requirement | C1_Sample@JDM | Softwaretest_FunctionalSoftwaretest | false |
| SW10-196 | The CAN ID request is based on J1939 PGN 0xDA with a destination address of 0xF5 and a source address from any other valid ECU address on the CAN bus for JDM modules. | agreed | requirement | C1_Sample@JDM | Softwaretest_FunctionalSoftwaretest | false |
| SW10-197 | The SCM shall use the address 0x18DAFEF5 for all CAN-ID responses from the JDM | agreed | requirement | C1_Sample@JDM | Softwaretest_FunctionalSoftwaretest | false |
| SW10-198 | The CAN ID response is based on the J1939 PGN 0xDB00 with a destination address equal to the requesting ECU and a source address of 0xF5 on the JDM. | agreed | information | C1_Sample@JDM | n/a | false |
| SW10-199 | The SCM shall use the address 0x18DBF5FE for all functional CAN-ID requests on the JDM | agreed | information | C1_Sample@JDM | n/a | false |
| SW10-200 | The SCM shall use the address 0x18DBFEF5 for all functional CAN-ID responses from the JDM | agreed | information | C1_Sample@JDM | n/a | false |
| SW10-1627 | ##### 1.1.1.1.2 KPM UDS Message Configuration | agreed | headline | n/a | n/a | false |
| SW10-1628 | The SCM shall use the address 0x18DAF7FE for all KPM CAN-ID requests | agreed | requirement | C1_Sample@KPM | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1629 | The CAN ID request is based on J1939 PGN 0xDA with a destination address of 0xF7 and a source address from any other valid ECU address on the CAN bus for KPM modules. | agreed | requirement | C1_Sample@KPM | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1630 | The SCM shall use the address 0x18DAFEF7 for all CAN-ID responses from the KPM | agreed | requirement | C1_Sample@KPM | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1631 | The CAN ID response is based on the J1939 PGN 0xDB00 with a destination address equal to the requesting ECU and a source address of 0xF7 on the KPM. | agreed | information | C1_Sample@KPM | n/a | false |
| SW10-1632 | The SCM shall use the address 0x18DBF7FE for all functional CAN-ID requests on the KPM | agreed | information | C1_Sample@KPM | n/a | false |
| SW10-1633 | The SCM shall use the address 0x18DBFEF7 for all functional CAN-ID responses from the KPM | agreed | information | C1_Sample@KPM | n/a | false |
| SW10-2293 | ##### 1.1.1.1.3 Modular Keypad 6 Button UDS Message Configuration | agreed | headline | n/a | n/a | false |
| SW10-2294 | The SCM shall use the address 0x18DAF6FE for all Modular Keypad 6-button CAN-ID requests | agreed | requirement | B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2295 | The CAN ID request is based on J1939 PGN 0xDA with a destination address of 0xF6 and a source address from any other valid ECU address on the CAN bus for Modular Keypad 6-button modules. | agreed | requirement | B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2296 | The SCM shall use the address 0x18DAFEF6 for all CAN-ID responses from the Modular Keypad 6-button | agreed | requirement | B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2297 | The CAN ID response is based on the J1939 PGN 0xDB00 with a destination address equal to the requesting ECU and a source address of 0xF6 on the Modular Keypad 6-button. | agreed | information | B1_Sample@MK6BTN | n/a | false |
| SW10-2298 | The SCM shall use the address 0x18DBF6FE for all functional CAN-ID requests on the Modular Keypad 6-button | agreed | information | B1_Sample@MK6BTN | n/a | false |
| SW10-2299 | The SCM shall use the address 0x18DBFEF6 for all functional CAN-ID responses from the Modular Keypad 6-button | agreed | information | B1_Sample@MK6BTN | n/a | false |
| SW10-4 | ## 1.2 Transport protocol | agreed | headline | n/a | n/a | false |
| SW10-171 | The SCM shall use UDS over CAN (ISO15765) to communicate all diagnostic messages with Marquardt's internal end of line test equipment | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1797 | When messages contain more than 8 bytes of data, the data shall be broken into 8 byte messages. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1798 | The first transmission of a multipacket message shall contain the total byte count of the message excluding the byte count data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1799 | The first byte of each transmission shall contain the packet number of the transmission. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1800 | The flow control of data shall be handled using the Vector CAN stack driver. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-5 | ## 1.3 Calibration | agreed | headline | n/a | n/a | false |
| SW10-172 | The SCM shall allow for calibration of individual PWM trim levels to adjust each Indicator and backlight channel during operation. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-6 | ## 1.4 Configuration | agreed | headline | n/a | n/a | false |
| SW10-175 | Viewing diagnostic messages in the SCM shall require Extended diagnostic mode access | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-176 | Programming of the SCM through diagnostic messages shall require programmer mode access | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-12 | ## 1.5 Diagnostic Messages | agreed | headline | n/a | n/a | false |
| SW10-1892 | The SCM shall support reading UDS messages only while in Extended Diagnostic Session. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1893 | The SCM shall support writing to UDS messages only while in Programmer Session. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1894 | The SCM will not support UDS outside of the production environment with exception to gaining access to the advanced UDS diagnostic sessions. | agreed | information | n/a | n/a | false |
| SW10-2192 | ### 1.5.1 Negative Response Code Conditions | agreed | headline | n/a | n/a | false |
| SW10-2193 | The ECU shall transmit the response code 0x12 when a requested sub-function is not supported by the ECU. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1912 | The ECU shall transmit the response code 0x13 when the data length of the request is does not match the expected data length within the ECU. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1913 | The ECU shall transmit the response code 0x14 when the message exceeds the limit of the transport protocol for the ECU. | agreed | requirement | obsolete | n/a | false |
| SW10-2186 | The ECU shall transmit the response code 0x22 when the ECU is unable to perform the operations request due to internal processing priorities. | agreed | requirement | obsolete | n/a | false |
| SW10-2187 | The ECU shall transmit the response code 0x31 when the value transmitted to the ECU is outside of the upper and lower boundary limits as specified. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1914 | The ECU shall transmit the response code 0x33 when Security Access information has not been provided and the ECU is still in a locked state. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2194 | The ECU shall transmit the response code 0x35 when a security access key has been transmitted with an incorrect value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-2195 | The ECU shall transmit the response code 0x36 when the number of attempts to gain security access has been exceeded. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-2196 | The ECU shall transmit the response code 0x37 when the timout for attempting security access after the number of access attempts were exceeded is still being enforced. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-2191 | The ECU shall transmit the response code 0x72 when an error has been detected while trying to write the data to memory within the ECU. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-177 | ### 1.5.2 Diagnostic Sessions | agreed | headline | n/a | n/a | false |
| SW10-206 | Access to the diagnostic session services will be done through diagnostic session control 0x10 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-179 | #### 1.5.2.1 Default Session | agreed | headline | n/a | n/a | false |
| SW10-182 | The application shall use Diagnostic Session type 0x01 for access to the default session. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-184 | The application shall send the positive response 50 01 and the appropriate data when the session is entered correctly | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1728 | The P2SM* value shall have a valid range of 0x0000 to 0xFFFF, 0 to 65535 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1729 | The P2SM* value shall provide a timer resolution of 1ms. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1745 | The P2SM* shall have a default value of 0x0032, 50 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1730 | The P2EM* value shall have a valid range of 0x0000 to 0xFFFF, 0 to 65535 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1731 | The P2EM* value shall provide a timer resolution of 10ms. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1744 | The P2EM* shall have a default value of 0x01F4, 500 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1764 | The P2SMH byte shall be the high byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1765 | The P2SML byte shall be the low byte value. | in work | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1766 | The P2EMH byte shall be the high byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1767 | The P2EML byte shall be the low byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-185 | The application shall send the negative response 7F 10 and the appropriate negative response code when a session error occurs. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-187 | The positive response code shall provide a 2 byte HEX dump and a 2 byte linear value with a multiplier of 10. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-221 | Negative response codes shall have the listed values: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1934 | The ECU shall transmit response code 0x12 when the requested function is not supported by the ECU. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1935 | The ECU shall transmit response code 0x13 when the data length of the request does not match the expected data length of the command. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1936 | The ECU shall transmit response code 0x22 when the ECU is unable to respond to the request. | agreed | requirement | obsolete | n/a | false |
| SW10-180 | #### 1.5.2.2 Programming Session | agreed | headline | n/a | n/a | false |
| SW10-1634 | Access to the programming session shall require security access | in work | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-204 | The application shall use Diagnostic Session type 0x02 for access to the programming session. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-207 | The application shall send the positive response 50 01 and the appropriate 4 bytes of data when the session is entered into correctly | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1732 | The P2SM* value shall have a valid range of 0x0000 to 0xFFFF, 0 to 65535 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1733 | The P2SM* value shall provide a timer resolution of 1ms. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1742 | The P2SM* value shall have a default value of 0x0032, 50 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1734 | The P2EM* value shall have a valid range of 0x0000 to 0xFFFF 0 to 65535 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1735 | The P2EM* value shall provide a timer resolution of 10ms. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1743 | The P2EM* value shall have a default value of 0x01F4, 500 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1768 | The P2SMH byte shall be the high byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1769 | The P2SML byte shall be the low byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1770 | The P2EMH byte shall be the high byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1771 | The P2EML byte shall be the low byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-208 | The application shall send the negative response 7F 10 and the appropriate negative response code when a session error occurs. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-209 | The positive response code shall provide a 2 byte HEX dump and a 2 byte linear value with a multiplier of 10. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-210 | Negative response codes shall have the listed values: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1931 | The ECU shall transmit the response code 0x12 when the requested function is not supported by the ECU. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1932 | The ECU shall transmit the response code 0x13 when the data received has the wrong data length. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1933 | The ECU shall transmit the response code 0x22 when the ECU is unable to respond to the request. | agreed | requirement | obsolete | n/a | false |
| SW10-2254 | The ECU shall transmit response code 0x33 when the ECU has not been unlocked using the security access routines. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-181 | #### 1.5.2.3 Extended Session | agreed | headline | n/a | n/a | false |
| SW10-1635 | Access to the extended session shall require security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-232 | The application shall use diagnostic session type 0x03 for access to the Extended diagnostic session. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-233 | The application shall send the positive response 50 03 and the appropriate 4 bytes of data when the session is entered into correctly. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1736 | The P2SM* value shall have a valid range of 0x0000 to 0xFFFF, 0 to 65535 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1737 | The P2SM* value shall provide a timer resolution of 1ms. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1740 | The P2SM* shall have a default value of: 0x0032, 50 decimal | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1738 | The P2EM* value shall have a valid range of 0x0000 to 0xFFFF, 0 to 65535 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1739 | The P2EM* value shall provide a timer resolution of 10ms. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1741 | The P2EM* shall have a default value of: 0x01F4, 500 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1772 | The P2SMH byte shall be the high byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1773 | The P2SML byte shall be the low byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1774 | The P2EMH byte shall be the high byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1775 | The P2EML byte shall be the low byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-234 | The application shall send the negative respinse 7F 10 and the appropriate negative response code when a session error occurs. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-235 | The positive response code shall provide a 2 byte hex dump and a 2 byte linear value with a multiplier or 10. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-236 | Negative response codes shall have the listed values: | agreed | requirement | obsolete | n/a | false |
| SW10-1927 | The ECU shall transmit response code 0x12 when the requested function is not supported by the ECU | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1928 | The ECU shall transmit response code 0x13 then the data received is of the incorrect length for the requested command. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1929 | The ECU shall transmit resonse code 0x22 when the ECU is unable to respond to the request. | agreed | requirement | obsolete | n/a | false |
| SW10-1930 | The ECU shall transmit response code 0x33 when the ECU has not been unlocked using the security access routines. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-178 | ### 1.5.3 Security Access | agreed | headline | n/a | n/a | false |
| SW10-251 | Access to the diagnostic Security Access services shall be through diagnostic session control 0x27. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2288 | The Security access seed shall have a minimum value of 0x00000000. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2289 | The Security access seed shall have a maximum value of 0xFFFFFFFE. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1561 | The Security access seed shall have a value of 0x00000000, 0 decimal, when the ECU is already unlocked. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1562 | The security access seed shall have a value of 0xFFFFFFFF, 4,294,967,295 decimal, when the ECU is unprogrammed. | agreed | requirement | obsolete | n/a | false |
| SW10-1563 | The security access seed shall have a valid unlocked key value range of 0x00000001 to 0xFFFFFFFE, 0 to 4,294,967,294 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-249 | #### 1.5.3.1 Seed Request | agreed | headline | n/a | n/a | false |
| SW10-250 | The application shall use session ID 0x01 to transmit the security access key to the requesting device. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-253 | The application shall send the positive response 67 01 with the 4 byte security access key data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1776 | the SEED0 byte shall contain the high byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1777 | The SEED3 byte shall contain the low byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-254 | The application shall send the negative response 7F 27 with the appropriate negative response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1637 | This sub-function shall use the following negative response codes: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1922 | The ECU shall transmit response code 0x12 when the ECU is unable to support the requested function. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1923 | The ECU shall transmit response code 0x13 when the data received by the ECU is of the incorrect data length. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1924 | The ECU shall transmit response code 0x22 when the ECU is unable to respond to the command | agreed | requirement | obsolete | n/a | false |
| SW10-1925 | The ECU shall transmit response code 0x24 when the KEY value has been received before the seed request has been transmitted. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1888 | While the Security Access timout is being enforces, the SCM shall respond to requests for security access with response code 0x37. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-265 | #### 1.5.3.2 Send Key | agreed | headline | n/a | n/a | false |
| SW10-2253 | The ECU requesting security access shall have a maximum of diag_SecAccess_ResponseTimeout to respond before the seed is invalid | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-266 | The application shall use session ID 0x02 to receive the security access key from the device attempting communications. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1778 | The KEY0 byte shall contain the high byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1779 | The KEY3 byte shall contain the low byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-267 | The application shall send the positive response 67 02 for a correct security access key | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-268 | The application shall send the negative response 7F 27 for an incorrect security access key with the appropriate negative response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1636 | This sub-function shall use the following negative response codes: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1916 | The ECU shall transmit response code 0x12 when the ECU is unable to respond to the requested function. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1917 | The ECU shall transmit response code 0x13 when the data length is incorrect for the command. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1918 | The ECU shall transmit response code 0x22 when the ECU is unable to respond to the requested command. | agreed | information | obsolete | n/a | false |
| SW10-1919 | The ECU shall transmit response code 0x24 when the ECU received the KEY value before the seed request was transmitted. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1921 | The ECU shall transmit response code 0x35 when the KEY value received does not match the calculated KEY that is expected based on the SEED value transmitted. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1889 | When the SCM has received xDIAG_LoginFailureCount requests for security access in succession, the ECU shall report response code 0x36. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1890 | Upon receiving xDIAG_LoginFailureCount requests, the SCM shall require a timeout of xDIAG_ResetTimeout before allowing any more attempts at security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2199 | After a power on reset of the ECU, the security access timeout shall reset. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-2200 | After a power on reset of the ECU, the security access login counter shall reset to 0. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-279 | #### 1.5.3.3 Security Seed/Key Algorithms | agreed | headline | n/a | n/a | false |
| SW10-1559 | The security seed shall be a randomly generated 4 byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1703 | The security seed shall have a minimum value of 0x00000001, 1 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1704 | The security seed shall have a maximum value of 0xFFFFFFFE, 4,294,967,294 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1560 | The security algorithm used to calculate the key value shall be: KEY = SEED^ CONST^ TABLE1[SEED& 0x00000007] where: KEY = Security Algorithm SEED = Random 4 Byte Value CONST = 0x7F9824E3 TABLE1[8] = { 0: 0x9E72A41B, 1: 0x4BC58935, 2: 0x78DA297C, 3: 0x563FF89A, 4: 0xFDB489EC, 5: 0x6E8990B4, 6: 0x0D5FC3A1, 7: 0x89F3B29F } | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-16 | ### 1.5.4 ECU Identification | agreed | headline | n/a | n/a | false |
| SW10-282 | #### 1.5.4.1 Data Diagnostic Identifier (DDI) | agreed | headline | n/a | n/a | false |
| SW10-394 | The data diagnostic Identifer shall use the access ID 0x0101 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-311 | ##### 1.5.4.1.1 Read | agreed | headline | n/a | n/a | false |
| SW10-391 | The application shall use the protocol service 22 01 01 for read access to the data Diagnostic Identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-393 | The application shall require extended session with security access in order to read the diagnostic data Identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-392 | The application shall return the positive response 62 01 01 (data) when the read request requirements have been fullfilled. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-395 | The application shall return the 2 byte Diagnostic Version defined as x_DiagnosticDataVersion | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1714 | Each byte of the diagnostic version data shall have a minimum value of 0x00, 0 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1715 | Each byte of the diagnostic version data shall have a maximum value of 0xFF, 255 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-396 | The application shall return the negative response 7F 22 with the 1 byte response code | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-397 | The application shall use the following negative response codes: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-312 | ##### 1.5.4.1.2 Write | agreed | headline | n/a | n/a | false |
| SW10-405 | The application shall use the protocol service 2E 01 01 and then the 2 byte data packet when receiving the write command from the diagnostic device | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1821 | Each byte of the diagnostic version data shall have a minimum value of 0x00, 0 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1822 | Each byte of the diagnostic version data shall have a maximum value of 0xFF, 255 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-406 | The application shall require programmer mode with security access in order to write the diagnostic Data Identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-407 | The application shall use the positive response 6E 01 01 to indicate that the command was received and processed correctly. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-409 | The application shall return the negative response 7F 2E with the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-410 | The application shall use the following negative write response codes: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-283 | #### 1.5.4.2 Development Data | agreed | headline | n/a | n/a | false |
| SW10-430 | The Development Data Identifier shall use the access ID 0x0100 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-313 | ##### 1.5.4.2.1 Read | agreed | headline | n/a | n/a | false |
| SW10-431 | The application shall use the protocol service 22 01 00 for read access to the Development Data identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-432 | The application shall require extended session with security access in order to read the Development Data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-433 | The application shall return the positive response 62 01 00 (data) when the read request requirements have been fullfilled. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-434 | The application shall return the following data: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1899 | The Base Value of the development data ID table represents the actual version information value. | agreed | information | n/a | n/a | false |
| SW10-1900 | The Response Value of the development data ID table represents the version information that will be reported by the ECU for the given Response Value. | agreed | information | n/a | n/a | false |
| SW10-1617 | All values for the 2-byte development data shall have a minimum value of 0x0001, 1 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1618 | All values for the 2-byte development data shall have a maximum value of 0xFFFF, 65535 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1782 | All byte data shall be intel format. example: OS1 = High Byte, OS2 = Low Byte | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-435 | The application shall return the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-436 | The application shall use the following negative read response codes: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-314 | ##### 1.5.4.2.2 Write | agreed | headline | n/a | n/a | false |
| SW10-444 | The application shall use the protocol service 2E 01 00 and then the following additional data: 2 Bytes - OS Version 2 Bytes - CAN Driver Version 2 Bytes - Network Management Version 2 Bytes - Diagnostic Module Version 2 Bytes - Transport Layer Version | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1823 | All values for the 2-byte development data shall have a minimum value of 0x0001, 1 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1824 | All values for the 2-byte development data shall have a maximum value of 0xFFFF, 65535 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-466 | The application shall require programmer mode with security access in order to write the development data | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1619 | The 2 byte development data will have a minimum value of 0x0001, 1 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1620 | The 2-byte development data will have a maximum value of 0xFFFF, 65535 decimal | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-467 | The application shall use the positive response code 6E 01 00 to indicate that the development data was received and processed correctly. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-445 | The application shall return the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-446 | The application shall use the following negative write response codes: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-284 | #### 1.5.4.3 Spare Part Number | agreed | headline | n/a | n/a | false |
| SW10-579 | The Spare Part Number Identifier shall use the access ID 0xF187 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-315 | ##### 1.5.4.3.1 Read | agreed | headline | n/a | n/a | false |
| SW10-468 | The application shall use the protocol service 22 F1 87 for read access to the spare part number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-469 | The application shall require extended session with security access in order to read the Spare Part Number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-470 | The application shall return the positive response 62 F1 87 (data) when the read request requirements have been fullfilled. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-471 | The application shall return the following data: 8 Bytes - Spare Part Number | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1701 | each byte of the spare part number shall have the following value range: | agreed | information | C1_Sample@JDM C1_Sample@KPM | n/a | false |
| SW10-1751 | The value byte SPN0 shall contain the HIGH byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1752 | The value in byte SPN7 shall contain the LOW byte value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1702 | The byte order for the spare part number shall use the intel byte order, meaning SPN7 is the LSB | agreed | information | C1_Sample@JDM C1_Sample@KPM | n/a | false |
| SW10-1897 | The Spare part number shall have a default value of 0xFF, 255 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1898 | The Spare part number shall be read from memory location EEP_MQ_CUST_PART_NO | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-472 | The application shall return the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-473 | The application shall use the following negative read response codes: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-286 | #### 1.5.4.4 ECU Identification | agreed | headline | n/a | n/a | false |
| SW10-581 | The ECU Identifier shall use the access ID 0xF189 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-319 | ##### 1.5.4.4.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1644 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-494 | The application shall use the protocol service 22 F1 89 for read access to the ECU identification. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-495 | The application shall require Extended Session Access in order to read the ECU Identification. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-496 | The application shall return the positive response 62 F1 89 (data) when the read request requirements have been fullfilled. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-497 | The application shall return the following data: 13 Bytes - Part Name | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1903 | The part name shall have a representation of the Binary Coded decimal values of the address claim data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1904 | Each byte of the ECU Identification shall be defined as follows: 0xWW, 0xYP, 0xPP, 0xPM, 0xMM, 0xME, 0xUU, 0xTT, 0xTV, 0xVV, 0xHH, 0xIR, 0xFF | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1905 | ECU Identification Read Example: Identity Number: 493665 Prod. Week = 15 Prod. Year = 16 (0 after MOD) Prod. Number = 2145 Maunfacturer ID: 709 ECU ID: 0 Function Instance: 0 Function: 135 Vehical System: 0 Vehical System Inst: 0 Industry Group: 0 Arbitrary Address Cap.: 1 Response Message: 0x15 02 14 50 70 90 00 13 50 00 00 01 FF | agreed | information | n/a | n/a | false |
| SW10-1710 | Each byte of the ECU Identification value shall have a minumim value of 0x00, 0 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1711 | Each byte of the ECU Identification value shall have a maximum value of 0xFF, 255 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1757 | The value in byte PN0 shall contain the HIGH byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1758 | The value in byte PN12 shall contain the LOW byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1759 | Flow control of the message shall be handled by the transport layer with the first byte of data signifying the location in the data packet. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-498 | The application shall return the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-499 | The application shall use the following negative read response codes: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-17 | ### 1.5.5 Stored Data | agreed | headline | n/a | n/a | false |
| SW10-38 | #### 1.5.5.1 MQ_ACL_ECUID | agreed | headline | n/a | n/a | false |
| SW10-582 | The MQ_ACL_ECUID shall allow access using the Identifier 0xFE00 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-321 | ##### 1.5.5.1.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1645 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | in work | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-614 | The application shall allow access to read the MQ_ACL_ECUID information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-615 | The application shall require the mode Extended Diagnostic Session to read the MQ_ACL_ECUID. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-616 | The application shall have a positive response of 62 FE 00 (Data) when all conditions have been met to read the ECUID data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-617 | The Application data returned shall consist of the following information: 3 Byte - ECU ID data | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1708 | The ECUID data shall have a minimum value of 0x000000, 0 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1709 | The ECUID data shall have a maximum value of 0x1FFFFF, 2097151 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1749 | The ECUID0 byte shall contain the HIGH byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1750 | The ECUID2 byte shall contain the LOW byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2201 | The ECUID shall have a default value of 0x000000. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-618 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-619 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-322 | ##### 1.5.5.1.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1681 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1351 | The application shall allow access to write the MQ_ACL_ECUID information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1832 | The ECUID data shall have a minimum value of 0x000000, 0 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1833 | The ECUID data shall have a maximum value of 0x1FFFFF, 2097151 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1834 | The ECUID0 byte shall contain the HIGH byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1835 | The ECUID2 byte shall contain the LOW byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1352 | The application shall require programmer access mode session to write the MQ_ACL_ECUID values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1353 | The application shall have a positive response of 6E FE 00 when all conditions have been met to write the ECUID data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1354 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1355 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1982 | The ECU shall transmit response code 0x13 when the data length of the command contains a value other than the expected value by the ECU. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1983 | The ECU shall transmit response code 0x31 when the values of data in the command are not within the defined minimum and maximum values for the datatype. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1984 | The ECU shall transmit response code 0x33 when ECU security is being enforced. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1985 | The ECU shall transmit response code 0x72 when the data transmitted to the ECU is not within the minimum and maximum values for the data type. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-287 | #### 1.5.5.2 ACL Function Instance | agreed | headline | n/a | n/a | false |
| SW10-583 | The ACL Function Instance data shall allow access using the Identifier 0xFD01 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-323 | ##### 1.5.5.2.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1646 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-628 | The application shall allow access to read the ACL Function Instance information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-629 | The application shall require the mode Extended Diagnostic Session to read the ACL Function Instance. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-630 | The application shall have a positive response of 62 FD 01 (Data) when all conditions have been met to read the Function Instance data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-631 | The Application data returned shall consist of the following information: 1 Byte - 5-Bit Function instance value | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1706 | The function instance shall have a minimum value of 0x00, 0 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1707 | The Function instance shall have a maximum value of 0x1F, 31 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2202 | The Function instance shall have a default value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-632 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-633 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-324 | ##### 1.5.5.2.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1682 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1356 | The application shall allow access to write the ACL Function Instance information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1836 | The function instance shall have a minimum value of 0x0, 0 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1837 | The Function instance shall have a maximum value of 0x1F, 31 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1357 | The application shall require programmer access mode session to write the ACL Function Instance values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1358 | The application shall have a positive response of 6E FD 01 when all conditions have been met to write the function instance data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1359 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1360 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-39 | #### 1.5.5.3 ACL Manufacturer Code | agreed | headline | n/a | n/a | false |
| SW10-584 | The ACL Manufacturer Code data shall allow access using the Identifier 0xFD02 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-325 | ##### 1.5.5.3.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1647 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-641 | The application shall allow access to read the manufacturer code information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-642 | The application shall require the mode Extended Diagnostic Session to read the manufacturer code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-643 | The application shall have a positive response of 62 FD 02 (Data) when all conditions have been met to read the manufacturer code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-644 | The Application data returned shall consist of the following information: 2 Byte - 11-Bit Manufacturer Code | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1716 | The manufacturer code shall have a minimum value of 0 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1717 | The manufacturer code shall have a maximum value of 0x7FF, 2047 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1747 | The MFG0 byte shall contain the HIGH byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1748 | The MFG1 byte shall contain the LOW byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2203 | The manufacturer code shall have a default value of 0x0008 (8 decimal) | in review | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-645 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-646 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-326 | ##### 1.5.5.3.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1683 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1361 | The application shall allow access to write the ACL Manufacturer Code information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1838 | The manufacturer code shall have a minimum value of 0 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1839 | The manufacturer code shall have a maximum value of 0x7FF, 2047 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1840 | The MFG0 byte shall contain the HIGH byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1841 | The MFG1 byte shall contain the LOW byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1362 | The application shall require programmer access mode session to write the ACL Manufacturer Code values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1363 | The application shall have a positive response of 6E FD 02 when all conditions have been met to write the manufacturer code data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1364 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1365 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-40 | #### 1.5.5.4 ACL ECU Instance | agreed | headline | n/a | n/a | false |
| SW10-585 | The ACL ECU Instance data shall allow access using the Identifier 0xFD03 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-327 | ##### 1.5.5.4.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1648 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-654 | The application shall allow access to read the ECU instance information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-655 | The application shall require the mode Extended Diagnostic Session to read the ECU instance data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-656 | The application shall have a positive response of 62 FD 03 (Data) when all conditions have been met to read the ECU instance. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-657 | The Application data returned shall consist of the following information: 1 Byte - 3 - Bit ECU Instance | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1718 | The ECU instance shall have a minimum value of 0. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1719 | The ECU instance shall have a maximum value of 7. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2204 | The ECU instance shall have a default value of 0. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-658 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-659 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-328 | ##### 1.5.5.4.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1684 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1366 | The application shall allow access to write the ACL ECU Instance information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1842 | The ECU instance shall have a minimum value of 0. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1843 | The ECU instance shall have a maximum value of 7. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1367 | The application shall require programmer access mode session to write the ACl ECU Instance values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1368 | The application shall have a positive response of 6E FD 03 when all conditions have been met to write the ECU Instance data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1369 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1370 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-41 | #### 1.5.5.5 ACL Function | agreed | headline | n/a | n/a | false |
| SW10-586 | The ACL Function data shall allow access using the Identifier 0xFD04 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-329 | ##### 1.5.5.5.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1649 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-667 | The application shall allow access to read the function information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-668 | The application shall require the mode Extended Diagnostic Session to read the function information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-669 | The application shall have a positive response of 62 FD 04 (Data) when all conditions have been met to read the ECUID data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-670 | The Application data returned shall consist of the following information: 1 Byte - Function | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1720 | The Function shall have a minimum value of 0x00, 0 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1721 | The Function shall have a maximum value of 0xFE, 254 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2205 | The Function shall have a default value of 0x87, 135 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-671 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-672 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-330 | ##### 1.5.5.5.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1685 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1371 | The application shall allow access to write the ACL Function information ising protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1844 | The Function shall have a minimum value of 0x00, 0 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1845 | The Function shall have a maximum value of 0xFE, 254 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1372 | The application shall require programmer access mode session to write the ACL Function values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1373 | The application shall have a positive response of 6E FD 04 when all conditions have been met to write the Function data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1374 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1375 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-43 | #### 1.5.5.6 ACL Vehicle System | agreed | headline | n/a | n/a | false |
| SW10-587 | The ACL Vehicle System data shall allow access using the Identifier 0xFD05 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-331 | ##### 1.5.5.6.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1650 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-680 | The application shall allow access to read the vehicle system information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-681 | The application shall require the mode Extended Diagnostic Session to read the vehicle system information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-682 | The application shall have a positive response of 62 FD 05 (Data) when all conditions have been met to read the vehicle system information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-683 | The Application data returned shall consist of the following information: 1 Byte - 7 - Bit Vehicle system information | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1722 | The Vehicle System shall have a minimum value of 0 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1723 | The vehicle System shall have a maximum value of 0x7F, 127 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2206 | The vehicle system shall have a default value of 0. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-684 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-685 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-332 | ##### 1.5.5.6.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1686 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1376 | The application shall allow access to write the ACL Vehicle System information ising protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1846 | The Vehicle System shall have a minimum value of 0 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1847 | The vehicle System shall have a maximum value of 0x7F, 127 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1377 | The application shall require programmer access mode session to write the vehicle system values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1378 | The application shall have a positive response of 6E FD 05 when all conditions have been met to write the vehicle system data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1379 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1380 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-44 | #### 1.5.5.7 ACL Industry Group | agreed | headline | n/a | n/a | false |
| SW10-588 | The ACL Industry Group data shall allow access using the Identifier 0xFD06 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-333 | ##### 1.5.5.7.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1651 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-693 | The application shall allow access to read the industry group information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-694 | The application shall require the mode Extended Diagnostic Session to read the industry group. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-695 | The application shall have a positive response of 62 FD 06 (Data) when all conditions have been met to read the industry group data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-696 | The Application data returned shall consist of the following information: 1 Byte - 3 Bit - Industry group | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1724 | The Industry Group shall have a minumum value of 0 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1725 | The Industry Group shall have a maximum value of 7 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2207 | The Industry Group shall have a default value of 0. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-697 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-698 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-334 | ##### 1.5.5.7.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1687 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1381 | The application shall allow access to write the ACL Industry Group information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1848 | The Industry Group shall have a minumum value of 0 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1849 | The Industry Group shall have a maximum value of 7 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1382 | The application shall require programmer access mode session to write the industry group values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1383 | The application shall have a positive response of 6E FD 06 when all conditions have been met to write the industry group data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1384 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1385 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-45 | #### 1.5.5.8 ACL Arbitrary Address Capable | agreed | headline | n/a | n/a | false |
| SW10-589 | The ACL Arbitrary Address Capable data shall allow access using the Identifier 0xFD07 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-335 | ##### 1.5.5.8.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1653 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-706 | The application shall allow access to read the AAC information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-707 | The application shall require the mode Extended Diagnostic Session to read the AAC. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-708 | The application shall have a positive response of 62 FD 07 (Data) when all conditions have been met to read the AAC data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-709 | The Application data returned shall consist of the following information: 1 Byte - 1-Bit Arbitrary address capable | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2208 | The Arbitrary address capable bit shall have a default value of 0. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-710 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-711 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-336 | ##### 1.5.5.8.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1688 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1386 | The application shall allow access to write the ACL Arbitrary Address Capable information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1850 | The Application data returned shall consist of the following information: 1 Byte - 1-Bit Arbitrary address capable | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1387 | The application shall require programmer access mode session to write the arbitrary address capable values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1388 | The application shall have a positive response of 6E FD 07 when all conditions have been met to write the arbitrary address capable data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1389 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1390 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-46 | #### 1.5.5.9 ACL Vehicle System Instance | agreed | headline | n/a | n/a | false |
| SW10-590 | The ACL Vehicle System Instance shall allow access using the Identifier 0xFD08 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-337 | ##### 1.5.5.9.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1654 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-719 | The application shall allow access to read the vehicle system instance information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-720 | The application shall require the mode Extended Diagnostic Session to read the vehicle system instance. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-721 | The application shall have a positive response of 62 FD 08 (Data) when all conditions have been met to read the vehicle system instance. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-722 | The Application data returned shall consist of the following information: 1 Byte - 4-Bit Vehicle System Instance | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1726 | The Vehicle System Instance shall have a minimum value of 0 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1727 | The Vehicle System Instance shall have a maximum value of 0xF, 15 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2209 | The vehicle system instance shall have a default value of 0. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-723 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-724 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-338 | ##### 1.5.5.9.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1689 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1391 | The application shall allow access to write the Vehicle System Instance information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1851 | The Vehicle System Instance shall have a minimum value of 0 (decimal). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1852 | The Vehicle System Instance shall have a maximum value of 0xF, 15 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1392 | The application shall require programmer access mode session to write the vehicle system instance values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1393 | The application shall have a positive response of 6E FD 08 when all conditions have been met to write the vehicle system instance data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1394 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1395 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-47 | #### 1.5.5.10 ACL System Address | agreed | headline | n/a | n/a | false |
| SW10-591 | The ACL System Address shall allow access using the Identifier 0xFD09 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-339 | ##### 1.5.5.10.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1655 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-732 | The application shall allow access to read the system address information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-733 | The application shall require the mode Extended Diagnostic Session to read the system address. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-734 | The application shall have a positive response of 62 FD 09 (Data) when all conditions have been met to read the system address data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-735 | The Application data returned shall consist of the following information: 1 Byte - System Address | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1783 | The system address data shall have a valid minimum value of 0x25, 37 Decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1784 | The system address data shall have a valid maximum value of 0xFD, 253 Decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1785 | The system address shall have a default value of 0xF5 for the JDM. | in review | requirement | C1_Sample@JDM | Softwaretest_Development | false |
| SW10-2329 | The system address shall have a default value of 0xF7 for the KPM. | in review | requirement | C1_Sample@KPM | Softwaretest_Development | false |
| SW10-2330 | The system address shall have a default value of 0x80 for the Modular Keypad. | in review | requirement | B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-736 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-737 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-340 | ##### 1.5.5.10.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1690 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1396 | The application shall allow access to write the System Address information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1853 | The system address data shall have a valid minimum value of 0x25, 37 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1854 | The system address data shall have a valid maximum value of 0xFD, 253 decimal. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1397 | The application shall require programmer access mode session to write the system address values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1398 | The application shall have a positive response of 6E FD 09 when all conditions have been met to write the system address data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1399 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1400 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-288 | #### 1.5.5.11 MQ_PWM_TABLE | agreed | headline | n/a | n/a | false |
| SW10-592 | The MQ_PWM_TABLE shall allow access using the Identifier 0xFD10 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1705 | The MQ_PWM_TABLE allows for the setting of the trim levels for each LED on the module | agreed | information | C1_Sample@JDM C1_Sample@KPM | n/a | false |
| SW10-341 | ##### 1.5.5.11.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1656 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-745 | The application shall allow access to read the PWM table information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-746 | The application shall require the mode Extended Diagnostic Session to read the PWM table. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-747 | The application shall have a positive response of 62 FD 10 (Data) when all conditions have been met to read the PWM table data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-748 | The Application data returned shall consist of the following information: 32 Byte - PWM Table | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1786 | Each byte of channel data (CHx) shall have a valid minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1787 | Each byte of channel data (CHx) shall have a valid maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2210 | Each byte of the channel data (CHx) shall have a default value of 0x7D(125). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-749 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-750 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-342 | ##### 1.5.5.11.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1691 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1401 | The application shall allow access to write the PWM Table information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1855 | Each byte of channel data (CHx) shall have a valid minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1856 | Each byte of channel data (CHx) shall have a valid maximum value of 0xFA. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2291 | The PWM adjustment value shall configure the adjustment offset of the PWM channel as it allies to a 100% commanded intensity level. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2292 | The PWM adjustment value shall adjust (+/-) the PWM duty cycle for the corresponding PWM channel by adjusting the PWM lookup percentage by the difference for the channel in the PWM Lookup table by the lookup percentage. Calculation: Actual PWM output (CH1) = PWMlower + ((LUXlevel - LUXlow) * ( (PWMhigh - PWMlow) / (LUXhigh - LUXlow) ) ) See Calculation sheet in Dimensions: M208910/SW_Development/Design/PWM_Calculation_Worksheet.xlsx | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1402 | The application shall require programmer access mode session to write the PWM table values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1403 | The application shall have a positive response of 6E FD 10 when all conditions have been met to write the PWM table data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1404 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1405 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-2302 | #### 1.5.5.12 MQ_LED_USE_TABLE | in review | headline | n/a | n/a | false |
| SW10-2303 | The MQ_LED_USE_TABLE shall allow access using the Identifier 0xFD11 | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2304 | The MQ_LED_USE_TABLE allows for the activation and de-activation of LED's in order to adjust for variant configurations of the modules. | in review | information | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-2305 | ##### 1.5.5.12.1 Read | in review | headline | n/a | n/a | false |
| SW10-2307 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2308 | The application shall allow access to read the LED Configuration table information using protocol service 0x22 with the appropriate identifier. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2309 | The application shall require the mode Extended Diagnostic Session to read the LED use table. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2310 | The application shall have a positive response of 62 FD 11 (Data) when all conditions have been met to read the PWM table data. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2311 | The Application data returned shall consist of the following information: 5 Byte - LED Use Table | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2312 | Each byte of channel data (LEDGrpx) shall have a valid minimum value of 0x00. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2313 | Each byte of channel data (LEDGrpx) shall have a valid maximum value of 0xFF. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2314 | Each byte of the channel data (LEDGrpx) shall have a default value of 0x00. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2327 | LED ON/OFF data within the 5-byte data packet shall have the following layout: | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2328 | Each 2-bit data group has the following values: | in review | information | n/a | n/a | false |
| SW10-2315 | The Application shall use the negative response 7F 22 and the 1 byte response code. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2316 | The negative response code for the Read information shall be: | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-2306 | ##### 1.5.5.12.2 Write | in review | headline | n/a | n/a | false |
| SW10-2317 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2318 | The application shall allow access to write the LED Use Table data using protocol service 0x2E with the appropriate identifier. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2319 | Each byte of channel data (LEDGrpx) shall have a valid minimum value of 0x00. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2320 | Each byte of channel data (LEDGrpx) shall have a valid maximum value of 0xFF. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2321 | The LED Use table will activate and deactivate each LED individually based on the following layout: | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2322 | Each 2-bit data group has the following values: | in review | information | n/a | n/a | false |
| SW10-2323 | The application shall require programmer access mode session to write the LED Use table values. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2324 | The application shall have a positive response of 6E FD 10 when all conditions have been met to write the LED Use table data. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2325 | The application shall use the negative response 7F 2E and the 1 byte response code. | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2326 | The negative response code for the write information shall be: | in review | requirement | C2_Sample@JDM C2_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-289 | #### 1.5.5.13 MQ_CUSTOMER_PN | agreed | headline | n/a | n/a | false |
| SW10-593 | The MQ_CUSTOMER_PN data shall allow access using the Identifier 0xFD61 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-343 | ##### 1.5.5.13.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1657 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-758 | The application shall allow access to read the customer part number information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-759 | The application shall require the mode Extended Diagnostic Session to read the customer part number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-760 | The application shall have a positive response of 62 FD 61 (Data) when all conditions have been met to read the customer part number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-761 | The Application data returned shall consist of the following information: 8 Byte - Customer Part Number | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1789 | Byte CPN0 shall contain the HIGH byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1790 | Byte CPN7 shall contain the LOW byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1801 | Each byte of data shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1802 | Each byte of data shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2211 | Each byte of the customer part number shall have a default value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-762 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-763 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-344 | ##### 1.5.5.13.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1692 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1406 | The application shall allow access to write the Customer Part Number information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1859 | Byte CPN0 shall contain the HIGH byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1860 | Byte CPN7 shall contain the LOW byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1857 | Each byte of data shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1858 | Each byte of data shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1407 | The application shall require programmer access mode session to write the customer part number values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1408 | The application shall have a positive response of 6E FD 61 when all conditions have been met to write the customer part number data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1409 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1410 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-290 | #### 1.5.5.14 MQ_MSI_PN | agreed | headline | n/a | n/a | false |
| SW10-594 | The MQ_MSI_PN data shall allow access using the Identifier 0xFD60 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-345 | ##### 1.5.5.14.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1658 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-771 | The application shall allow access to read the Marquardt part number information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-772 | The application shall require the mode Extended Diagnostic Session to read the Marquardt part number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-773 | The application shall have a positive response of 62 FD 60 (Data) when all conditions have been met to read the Marquardt part number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-774 | The Application data returned shall consist of the following information: 8 Byte - Marquardt Part Number | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1791 | Byte MPN0 shall contain the HIGH byte data | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1792 | Byte MPN7 shall contain the LOW byte data | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1803 | Each byte of the MSI part number shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1804 | Each byte of the MSI part number shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2212 | Each byte of the MSI part number shall have a default value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-775 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-776 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-347 | ##### 1.5.5.14.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1693 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1411 | The application shall allow access to write the MSI Part Number information ising protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1861 | Byte MPN0 shall contain the HIGH byte data | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1862 | Byte MPN7 shall contain the LOW byte data | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1863 | Each byte of the MSI part number shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1864 | Each byte of the MSI part number shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1412 | The application shall require programmer access mode session to write the MSI part number values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1413 | The application shall have a positive response of 6E FD 60 when all conditions have been met to write the MSI part number data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1414 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1415 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-291 | #### 1.5.5.15 MQ_ICT_TB | agreed | headline | n/a | n/a | false |
| SW10-595 | The MQ_ICT_TB data shall allow access using the Identifier 0xFE10 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-348 | ##### 1.5.5.15.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1659 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-784 | The application shall allow access to read the ICT test byte information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-785 | The application shall require the mode Extended Diagnostic Session to read the ICT test byte. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-786 | The application shall have a positive response of 62 FE 10 (Data) when all conditions have been met to read the ICT test byte. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-787 | The Application data returned shall consist of the following information: 1 Byte - ICT Test Byte | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1805 | The ICT test byte shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1806 | The ICT test byte shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2213 | The ICT test byte shall have a default value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-788 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-789 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-349 | ##### 1.5.5.15.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1694 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1416 | The application shall allow access to write the ICT test byte information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1865 | The ICT test byte shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1866 | The ICT test byte shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1417 | The application shall require programmer access mode session to write the ICT test byte values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1418 | The application shall have a positive response of 6E FE 10 when all conditions have been met to write the ICT test byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1419 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1420 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-292 | #### 1.5.5.16 MQ_EOL_TB | agreed | headline | n/a | n/a | false |
| SW10-596 | The MQ_EOL_TB data shall allow access using the Identifier 0xFE11 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-350 | ##### 1.5.5.16.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1660 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-797 | The application shall allow access to read the EOL test byte information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-798 | The application shall require the mode Extended Diagnostic Session to read the EOL test byte. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-799 | The application shall have a positive response of 62 FE 11 (Data) when all conditions have been met to read the EOL test byte. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-800 | The Application data returned shall consist of the following information: 1 Byte - EOL test byte | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1807 | The EOL test byte value shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1808 | The EOL test byte value shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2214 | The EOL test byte shall have a default value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-801 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-802 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-351 | ##### 1.5.5.16.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1695 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1421 | The application shall allow access to write the EOL Test Byte information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1867 | The EOL test byte value shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1868 | The EOL test byte value shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1422 | The application shall require programmer access mode session to write the EOL Test Byte values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1423 | The application shall have a positive response of 6E FE 11 when all conditions have been met to write the EOL Test Byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1424 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1425 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-293 | #### 1.5.5.17 MQ EOL ITAC TraceNumber | agreed | headline | n/a | n/a | false |
| SW10-597 | The MQ EOL ITAC TraceNumber data shall allow access using the Identifier 0xFE12 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-352 | ##### 1.5.5.17.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1661 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-810 | The application shall allow access to read the ITAC traceability number using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-811 | The application shall require the mode Extended Diagnostic Session to read the ITAC traceability number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-812 | The application shall have a positive response of 62 FE 12 (Data) when all conditions have been met to read the ITAC traceability number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-813 | The Application data returned shall consist of the following information: 16 Byte - ITAC traceability number | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1795 | Each byte of the ITAC value shall have a valid minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1796 | Each byte of the ITAC value shall have a valid maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1793 | Byte ITAC0 shall contain the HIGH byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1794 | BYTE ITAC15 shall contain the LOW byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2215 | Each byte of the ITAC serial number shall have a default value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-814 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-815 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-354 | ##### 1.5.5.17.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1696 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1426 | The application shall allow access to write the ITAC traceability information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1869 | Each byte of the ITAC value shall have a valid minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1870 | Each byte of the ITAC value shall have a valid maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1871 | Byte ITAC0 shall contain the HIGH byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1872 | BYTE ITAC15 shall contain the LOW byte data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1427 | The application shall require programmer access mode session to write the ITAC traceability values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1428 | The application shall have a positive response of 6E FE 12 when all conditions have been met to write the ITAC data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1429 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1430 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-294 | #### 1.5.5.18 MQ_MSI_SN | agreed | headline | n/a | n/a | false |
| SW10-598 | The MQ_MSI_SN shall allow access using the Identifier 0xFE13 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-355 | ##### 1.5.5.18.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1662 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-823 | The application shall allow access to read the Marquardt serial number information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-824 | The application shall require the mode Extended Diagnostic Session to read the Marquardt serial number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-825 | The application shall have a positive response of 62 FE 13 (Data) when all conditions have been met to read the Marquardt serial number. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-826 | The Application data returned shall consist of the following information: 8 Byte - Marquardt Serial Number | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1809 | Each byte of the Marquardt serial number shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1810 | Each byte of the Marquardt serial number shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2216 | Each byteof the MSI Serial Number shall have a default value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1811 | Byte MSN0 shall contain the high byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1812 | Byte MSN7 shall contain the low byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-827 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-828 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-356 | ##### 1.5.5.18.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1697 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1431 | The application shall allow access to write the MSI Serial Number information using protocol service 0x2E with the appropriate identifier. | in work | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1873 | Each byte of the Marquardt serial number shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1874 | Each byte of the Marquardt serial number shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1875 | Byte MSN0 shall contain the high byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1876 | Byte MSN7 shall contain the low byte information. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1432 | The application shall require programmer access mode session to write the MSI Serial Number values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1433 | The application shall have a positive response of 6E FE 13 when all conditions have been met to write the serial number data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1434 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1435 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-295 | #### 1.5.5.19 MQ_SWV | agreed | headline | n/a | n/a | false |
| SW10-599 | The MQSWV data shall allow access using the Identifier 0xFE20 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-357 | ##### 1.5.5.19.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1663 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-836 | The application shall allow access to read the Marquardt software version information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-837 | The application shall require the mode Extended Diagnostic Session to read the Marquardt software version. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-838 | The application shall have a positive response of 62 FE 20 (Data) when all conditions have been met to read the software version. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-839 | The Application data returned shall consist of the following information: 3 Byte - Marquardt Software Version | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1813 | Each byte of the Marquardt software version shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1814 | Each byte of the Marquardt software version shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2217 | Each byte of the Marquardt software version shall have a default value equal to the software version of the stored software file in Dimensions. ex. File Name in Dimensions: M208910_C1.1_V_170900.S19 SWV Byte 0: 0x11 (17) SWV Byte 1: 0x09 (09) SWV Byte 2: 0x00 (00) | in review | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1815 | Each byte of the Marquardt software version shall contain unique information where: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-840 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-841 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-358 | ##### 1.5.5.19.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1698 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1436 | The application shall allow access to write the Software verison information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1877 | Each byte of the Marquardt software version shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1878 | Each byte of the Marquardt software version shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1879 | Each byte of the Marquardt software version shall contain unique information where: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1437 | The application shall require programmer access mode session to write the software version values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1438 | The application shall have a positive response of 6E FE 20 when all conditions have been met to write the software version data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1439 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1440 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-296 | #### 1.5.5.20 MQ_HWV | agreed | headline | n/a | n/a | false |
| SW10-600 | The MQ_HWV data shall allow access using the Identifier 0xFE21 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-359 | ##### 1.5.5.20.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1664 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-849 | The application shall allow access to read the Marquardt hardware version information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-850 | The application shall require the mode Extended Diagnostic Session to read the hardware version. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-851 | The application shall have a positive response of 62 FE 21 (Data) when all conditions have been met to read the hardware version. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-852 | The Application data returned shall consist of the following information: 3 Byte - Marquardt hardware version | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1816 | Each byte of the Marquardt Hardware version shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1817 | Each byte of the Marquardt hardware version shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2218 | Each byte of the Marquardt hardware version shall have a default value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1818 | Each byte shall contain a unique value where: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-853 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-854 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-360 | ##### 1.5.5.20.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1699 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1441 | The application shall allow access to write the hardware version information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1880 | Each byte of the Marquardt Hardware version shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1881 | Each byte of the Marquardt hardware version shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1442 | The application shall require programmer access mode session to write the hardware version values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1443 | The application shall have a positive response of 6E FE 21 when all conditions have been met to write the hardware version data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1444 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1445 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-297 | #### 1.5.5.21 MQ_Variant | agreed | headline | n/a | n/a | false |
| SW10-601 | The MQ_Variant data shall allow access using the Identifier 0xFE30 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2301 | Variant configurations are stored in Dimensions within the document: M208910/SW Development/Design/SCM_Memory_Map.xlsx -> Variant Coding | agreed | information | n/a | n/a | false |
| SW10-361 | ##### 1.5.5.21.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1665 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-862 | The application shall allow access to read the variant code information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-863 | The application shall require the mode Extended Diagnostic Session to read the variant code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-864 | The application shall have a positive response of 62 FE 30 (Data) when all conditions have been met to read the variant code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-865 | The Application data returned shall consist of the following information: 1 Byte - Variant Code | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1819 | The variant code shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1820 | The variant code shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2219 | The variant code shall have a default value of 0x01. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-866 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-867 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-362 | ##### 1.5.5.21.2 Write | agreed | headline | n/a | n/a | false |
| SW10-1700 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1446 | The application shall allow access to write the variant information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1882 | The variant code shall have a minimum value of 0x00. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1883 | The variant code shall have a maximum value of 0xFF. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-1447 | The application shall require programmer access mode session to write the variant values. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1448 | The application shall have a positive response of 6E FE 30 when all conditions have been met to write the variant data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1449 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1450 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-2257 | #### 1.5.5.22 MQ_Batt_Calibration | agreed | headline | n/a | n/a | false |
| SW10-2260 | The battery voltage calibration diagnostic is used to program in the required offset for the battery voltage monitor to achieve the appropriate tolerance. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-2261 | The MQ_Batt_Calibration shall allow access using the Identifier 0xFD74 | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2258 | ##### 1.5.5.22.1 Read | agreed | headline | n/a | n/a | false |
| SW10-2262 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2263 | The application shall allow access to read the indicator status information using the protocol service 0x22 with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2264 | The application shall have a positive response of 62 FD 74 (Data) when all conditions have been met to read the current indicator status. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2265 | The application data returned shall consist of the following information: 1 Byte - Voltage Offset Value | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2266 | The application shall have a minimum value of -128. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2267 | The application shall have a maximum value of 127. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2268 | The offset data representation is the indicated value divided by 10 (value/10). ex: value = 98 Offset : 98/10 = 9.8V | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-2271 | The application shall respond to a read request with the configured voltage compensation value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2272 | The default voltage calibration value shall be 0. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2273 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2274 | The negative response code for the Read information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-2259 | ##### 1.5.5.22.2 Write | agreed | headline | n/a | n/a | false |
| SW10-2275 | Access to write diagnostic sub-function shall require the ECU to be in programmer mode with security access. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2276 | The application shall allow access to write the voltage input information using protocol service 0x2E with the appropriate identifier. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2277 | The voltage input value shall have a minimum value of 100 (0x64). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2278 | The voltage input value shall have a maximum value of 250 (0xFA). | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2286 | The voltage input value shall indicate the test voltage being supplied by the end of line tester. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-2290 | The voltage range for testing shall be between 12V and 24V for calibration | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-2283 | The voltage input value is indicated by the offset value divided by 10 (value/10). ex. Value = 98 Offset : 98/10 = 9.8V | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-2284 | When calculating the voltage offset, the application shall calculate the difference between the measured voltage on the analog bus and the voltage provided by the diagnostic. Voltage Offset = Input Voltage - Measured Voltage | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2285 | When the calculation is preformed, the application shall prevent overflow errors from occuring. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2287 | The voltage offset shall apply to all voltage measurements reported on the CAN bus. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_Development | false |
| SW10-2279 | The application shall require programmer access mode session to write the voltage offset value. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2280 | The application shall have a positive response of 6E FD 74 when all conditions have been met to write the variant data. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2281 | The application shall use the negative response 7F 2E and the 1 byte response code. | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2282 | The negative response code for the write information shall be: | agreed | requirement | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-247 | ### 1.5.6 IO Control | agreed | headline | n/a | n/a | false |
| SW10-309 | #### 1.5.6.1 MQ_IND_CONTROL | agreed | headline | n/a | n/a | false |
| SW10-1348 | The MQ_IND_Control shall allow access using the identifier 0xFD21. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-386 | ##### 1.5.6.1.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1666 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1018 | The application shall allow access to read the indicator status information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1019 | The application shall require the mode Extended Diagnostic Session to read the current indicator status. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1020 | The application shall have a positive response of 62 FD 21 (Data) when all conditions have been met to read the current indicator status. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1021 | The Application data returned shall consist of the following information: 8 Byte - Indicator Status register | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2220 | The read service data shall be in the format of: | agreed | information | obsolete | n/a | false |
| SW10-2221 | Each 2 bit value is used to control a single indicator on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2222 | Any locations marked Rsrvd shall have the value 0x3 indicating that they are reserved. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2223 | The control modification service data fields use the following valid values to control the Indicators on the ECU: | agreed | information | obsolete | n/a | false |
| SW10-2252 | Any indicator that is not supported on the ECU shall return the value 0x0 for the OFF state. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1022 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1023 | The negative response code for the Read information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-387 | ##### 1.5.6.1.2 Return Control | agreed | headline | n/a | n/a | false |
| SW10-1667 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1506 | The application shall allow access to return control of indicators using protocol service 0x2F with the appropriate identifier and control value. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1507 | The return control modification service shall have a control value of 0x00. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2255 | Upon receipt of the return control function, The ECU shall reset the indictor states to the last known command setting from the Indicator Command received. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1508 | The return control modification service shall have a positive response value of 6F FD 21 (data) when all conditions have been met to return control of the idicators to the ECU. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1509 | The return data from a positive response shall consist of the 8 byte indicator status register. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2224 | The read service data shall be in the format of: | agreed | information | obsolete | n/a | false |
| SW10-2225 | Each 2 bit value is indicate the status of a single indicator on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2226 | Any locations marked Rsrvd shall have the value 0x3 indicating that they are Unavailable. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2227 | The control response data fields use the following valid values to indicate current state of the Indicators on the ECU: | agreed | information | obsolete | n/a | false |
| SW10-1510 | The application shall have a negative response of 7F 2F and the 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1511 | The negative response code for the return control information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1047 | ##### 1.5.6.1.3 Reset | agreed | headline | n/a | n/a | false |
| SW10-1668 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1512 | The application shall allow access to the reset control of the indicators using the protocol service 0x2F with the appropriate identifier and control value. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1513 | The reset modification service shall have a control value of 0x01. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1514 | The reset modification service shall have a positive response value of 6F FD 21 (data) when all conditions have been met to return control of the indicators to the ECU. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1515 | The return data from a positive response shall consist of the 8 byte indicator status register. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2228 | The read service data shall be in the format of: | agreed | information | obsolete | n/a | false |
| SW10-2229 | Each 2 bit value is used to indicate the status of a single indicator on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2230 | Any locations marked Rsrvd shall have the value 0x3 indicating that they are unavailable. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2231 | The control modification service data fields use the following valid values to control the Indicators on the ECU: | agreed | information | obsolete | n/a | false |
| SW10-1516 | The application shall have a negative response of 7F 2F and the 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1517 | The negative response code for the reset control information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1048 | ##### 1.5.6.1.4 Freeze | agreed | headline | n/a | n/a | false |
| SW10-1669 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1518 | The application shall allow access to the freeze control of the indicators using the protocol service 0x2F with the appropriate identifier and control value. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1519 | The freeze modification service shall have a control value of 0x02. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1520 | The freeze modification service shall have a positive response value of 6F FD 21 (data) when all conditions have been met to return control of the indicators to the ECU. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1521 | The return data from a positive response shall consist of the 8 byte indicator status register. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2232 | The read service data shall be in the format of: | agreed | information | obsolete | n/a | false |
| SW10-2233 | Each 2 bit value is used to indicate the value of a single indicator on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2234 | Any locations marked Rsrvd shall have the value 0x3 indicating that they are unavailable. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2235 | The control modification service data fields use the following valid values to control the Indicators on the ECU: | agreed | information | obsolete | n/a | false |
| SW10-1522 | The application shall have a negative response of 7F 2F and a 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1523 | The negative response code for the reset contol information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1049 | ##### 1.5.6.1.5 Control | agreed | headline | n/a | n/a | false |
| SW10-1670 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1524 | The application shall allow access to the control modification service of the indicators using the protocol service 0x2F with the appropriate identifier, data format and control value. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1525 | The control modification service shall have a control value of 0x03. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1526 | The control modification service data input shall be in the format of: | agreed | information | obsolete | n/a | false |
| SW10-1621 | The indicator contol modification service allows for changing the output status of each indicator on the ECU individually while UDS diagnostics are active. | agreed | information | obsolete | n/a | false |
| SW10-1622 | Each 2 bit value is used to control a single indicator on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2197 | Any locations marked Rsrvd shall have the value 0x3 indicating that they are reserved. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1531 | The control modification service data fields use the following valid values to control the Indicators on the ECU: | agreed | information | obsolete | n/a | false |
| SW10-1527 | The control modification service shall have a positive response value of 6F FD 21 (data) when all conditions have been met to control the status of the indicators. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1528 | The return data from a positive response shall consist of the 8 byte indicator status register. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1529 | The application shall have a negative response of 7F 2F and a 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1530 | The negative response code for the reset control information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-310 | #### 1.5.6.2 MQ_BL_CONTROL | agreed | headline | n/a | n/a | false |
| SW10-1347 | The MQ_BL_Control shall allow access using the identifier 0xFD20. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-388 | ##### 1.5.6.2.1 Read | agreed | headline | n/a | n/a | false |
| SW10-1671 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1005 | The application shall allow access to read the backlight status information using protocol service 0x22 with the appropriate identifier. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1006 | The application shall require the mode Extended Diagnostic Session to read the backlight status. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1007 | The application shall have a positive response of 62 FD 20 (Data) when all conditions have been met to read the backlight status. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1008 | The Application data returned shall consist of the following information: 1 Byte - Backlight status | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2248 | The service indication state data shall be in the format of: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2249 | Each 2 bit value is used to indicate the state of all backlighting on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2250 | Any location marked as Rsvrd shall have the value 0x3 indicating the feature is unavailable. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2251 | The control modification service data fields use have the following values to control the backlighting state: | agreed | information | obsolete | n/a | false |
| SW10-1009 | The Application shall use the negative response 7F 22 and the 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1010 | The negative response code for the Read information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-390 | ##### 1.5.6.2.2 Return Control | agreed | headline | n/a | n/a | false |
| SW10-1672 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1532 | The application shall allow access to return control of backlighting using protocol service 0x2F with the appropriate identifier and control value. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1533 | The return control modification service shall have a control value of 0x00. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2256 | Upon receipt of the return control function, The ECU shall reset the indictor states to the last known command setting from the Indicator Command received. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1534 | The return control modification service shall have a positive response value of 6F FD 20 (data) when all conditions have been met to return control of the idicators to the ECU. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1535 | The return data from a positive response shall consist of the 1 byte indicator status register. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2236 | The service indication state data shall be in the format of: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2238 | Each 2 bit value is used to indicate the state of all backlighting on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2239 | Any location marked as Rsvrd shall have the value 0x3 indicating the feature is unavailable. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2237 | The control modification service data fields use have the following values to control the backlighting state: | agreed | information | obsolete | n/a | false |
| SW10-1536 | The application shall have a negative response of 7F 2F and the 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1537 | The negative response code for the return control information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1044 | ##### 1.5.6.2.3 Reset | agreed | headline | n/a | n/a | false |
| SW10-1673 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1538 | The application shall allow access to the reset control of the backlighting using the protocol service 0x2F with the appropriate identifier and control value. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1539 | The reset modification service shall have a control value of 0x01. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1540 | The reset modification service shall have a positive response value of 6F FD 20 (data) when all conditions have been met to return control of the indicators to the ECU. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1541 | The return data from a positive response shall consist of the 1 byte indicator status register. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2240 | The service indication state data shall be in the format of: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2241 | Each 2 bit value is used to indicate the state of all backlighting on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2242 | Any location marked as Rsvrd shall have the value 0x3 indicating the feature is unavailable. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2243 | The control modification service data fields use have the following values to control the backlighting state: | agreed | information | obsolete | n/a | false |
| SW10-1542 | The application shall have a negative response of 7F 2F and the 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1543 | The negative response code for the reset control information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1045 | ##### 1.5.6.2.4 Freeze | agreed | headline | n/a | n/a | false |
| SW10-1674 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1544 | The application shall allow access to the freeze control of the backlighting using the protocol service 0x2F with the appropriate identifier and control value. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1545 | The freeze modification service shall have a control value of 0x02. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1546 | The freeze modification service shall have a positive response value of 6F FD 20 (data) when all conditions have been met to return control of the indicators to the ECU. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1547 | The return data from a positive response shall consist of the 1 byte indicator status register. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2244 | The service indication state data shall be in the format of: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2245 | Each 2 bit value is used to indicate the state of all backlighting on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2246 | Any location marked as Rsvrd shall have the value 0x3 indicating the feature is unavailable. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-2247 | The control modification service data fields use have the following values to control the backlighting state: | agreed | information | obsolete | n/a | false |
| SW10-1548 | The application shall have a negative response of 7F 2F and a 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1549 | The negative response code for the reset contol information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1046 | ##### 1.5.6.2.5 Control | agreed | headline | n/a | n/a | false |
| SW10-1675 | Access to this diagnostic sub-function shall require extended mode with security access to read data. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1550 | The application shall allow access to the control modification service of the backlighting using the protocol service 0x2F with the appropriate identifier, data format and control value. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1551 | The control modification service shall have a control value of 0x03. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1552 | The control modification service data input shall be in the format of: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1624 | The backlight contol modification service allows for changing the output status of the backlights on the ECU while UDS diagnostics are active. | agreed | information | obsolete | n/a | false |
| SW10-1625 | Each 2 bit value is used to control all backlighting on the ECU. | agreed | information | obsolete | n/a | false |
| SW10-2198 | Any location marked as Rsvrd shall have the value 0x3 indicating the feature is reserved. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1553 | The control modification service data fields use have the following values to control the backlighting state: | agreed | information | obsolete | n/a | false |
| SW10-1554 | The control modification service shall have a positive response value of 6F FD 20 (data) when all conditions have been met to control the status of the indicators. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1555 | The return data from a positive response shall consist of the 1 byte indicator status register. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1556 | The application shall have a negative response of 7F 2F and a 1 byte response code. | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest | false |
| SW10-1557 | The negative response code for the reset control information shall be: | agreed | requirement | obsolete | Softwaretest_FunctionalSoftwaretest Softwaretest_Development | false |
| SW10-1565 | ### 1.5.7 Negative Response Codes | agreed | headline | n/a | n/a | false |
| SW10-2300 | #### 1.5.7.1 NRC 0x11 - | agreed | n/a | n/a | n/a | false |
| SW10-1566 | #### 1.5.7.2 NRC 0x12 - Sub Function Not Supported | agreed | headline | n/a | n/a | false |
| SW10-1595 | The sub-function not supported response code indicates that the requested sub-function of a service is not supported. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1567 | #### 1.5.7.3 NRC 0x13 - Incorrect Message Length or Invalid Format | agreed | headline | n/a | n/a | false |
| SW10-1596 | The Incorrect Message Length or Invalid Format response indicates that the data transmitted to the ECU does not meet the required parameters to be acted upon. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1573 | #### 1.5.7.4 NRC 0x22 - Conditions Not Correct | agreed | headline | n/a | n/a | false |
| SW10-1599 | The conditions not correct response indicates that the ECU is missing the pre-requisite conditions to fullfil the requested action. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1895 | #### 1.5.7.5 NRC 0x24 - Request Sequence Error | agreed | headline | n/a | n/a | false |
| SW10-1896 | The request sequence error response can indicate that the programming process is incomplete when the request was received or that the Request Download or Request upload services are not active. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1577 | #### 1.5.7.6 NRC 0x31 - Request out of Range | agreed | headline | n/a | n/a | false |
| SW10-1603 | The Request Out-Of-Range response indicates that the requested operation is not within the ECU boundary limits for the data being modified by the request. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1578 | #### 1.5.7.7 NRC 0x33 - Security Access Denied | agreed | headline | n/a | n/a | false |
| SW10-1604 | The Security Access Denied response indicates that the ECU detected that the security access requirements were not met in order to continue with the operation. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1579 | #### 1.5.7.8 NRC 0x35 - Invalid Key | agreed | headline | n/a | n/a | false |
| SW10-1605 | The Invalid Key response indicates that the security key sent by the client does not match the required security key contained within the ECU. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1580 | #### 1.5.7.9 NRC 0x36 - Exceeded Number of Access Attempts | agreed | headline | n/a | n/a | false |
| SW10-1606 | The Exceeded Number of Access Attempts response indicates that the number of attempts to gain security access to the ECU has been exceeded. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1581 | #### 1.5.7.10 NRC 0x37 - Required Time Delay not Expired | agreed | headline | n/a | n/a | false |
| SW10-1607 | The required Time Delay not Expired response indicates that the time between failed attempts to gain security access to the ECU has not yet expired. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1584 | #### 1.5.7.11 NRC 0x72 - General Programming failure | agreed | headline | n/a | n/a | false |
| SW10-1610 | The General Programming Failure response indicates that there was an error while erasing or programming the ECU's non-volitile memory. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1587 | #### 1.5.7.12 NRC 0x7E - Sub-Function not supported in active session | agreed | headline | n/a | n/a | false |
| SW10-1614 | The Sub-Function not supported in active session indicates that the ECU will not respond because the request is not part of the current session. | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |
| SW10-1349 | # 2 Abbreviations | agreed | headline | n/a | n/a | false |
| SW10-1350 | All abbreviations are located at: SSC/Caterpillar/M2089/M208910_HEX_SCM/SRS/Working Modules/08_Abbreviations_en | agreed | information | C1_Sample@JDM C1_Sample@KPM B1_Sample@MK6BTN | n/a | false |